# Well Identification:

# Overview

The Mukluk 1 was spud as an exploratory well on November 1st, 1983 and located in the Beaufort Sea off the North Slope. The original wellbore was drilled to total depth without incident. The operator reported hydrocarbons discovered at this location, and the well was plugged and abandoned. The analytical data collection program included well logging by Schlumberger, coring, and drill cutting samples collected by Baroid.

# Geologic Intervals used for Analysis:

# Logging Runs and Parameters:

# Cored Intervals and Sample Analysis:

# Log Discussion:

The Mukluk 1 well was drilled and logged with water-based drilling fluid containing Barite weighting material to total depth. Subsequent borehole sections were drilled with additional Barite to increase the borehole fluid pressure overbalance. All borehole sections required environmental corrections for hole size, temperature, pressure, and mud weight additives.

# Environmental Corrections:

The Schlumberger 2000 Edition chartbook was used to correct the logs for borehole size, temperature, pressure, and drilling mud additives. The Gamma Ray log was corrected using chart GR-1. Compensated Neutron log was corrected using Por-14c and Por -14d. Dual Laterolog Resistivity logs were corrected using Rcor-2c and invasion corrected using Rint-9b. Dual Induction logs were corrected using Rcor-4a and invasion corrected using Rint-10.

Significant caliper enlargements were observed in various sections of the well, in cases where the borehole caliper readings were above the correction charts, the maximum chart correction was applied, however these corrections under estimate the true formation measurement.

The bulk density measurement was the most environmentally affected log in the dataset, where the density log readings measured drilling fluid when the caliper reading exceed 16 inches. Repair of the density log utilized a Gardner et al. (1974) sonic to density transform.

## Observations Logged Interval

Observed minor caliper readings in the well where density log was affected, the logged interval showed the bulk density required some editing using the Gardner1 density transform. Sonic log data was compared to the Faust4 velocity transform to correct anomalies for any borehole washouts. Logged intervals where the bulk density was not present the delta-t sonic was used as the porosity model input to the final computed results.

## References

- Gardner et al., 1974, Formation velocity and density—the diagnostic basics for stratigraphic traps Geophysics, 39 (6) (1974), pp. 770-780

- Graton, L. C., and H. J. Fraser, 1935, Systematic packing of spheres with particular reference to porosity and permeability: Journal of Geology, v. 43, p. 785–909, DOI: 10.1086/jg.1935.43.issue-8

- Carmichael, R.S. ed. 1982. Handbook of Physical Properties of Rocks, Vol. 2, 1-228. Boca Raton, Florida: CRC Press Inc.

- L. Y. Faust, “A Velocity Function Including Lithologic Variation,” Geophysics, Vol. 18, No. 2, 1953, pp. 271-288.

# Summation Report:

Reservoir summary cut off values used were porosity greater than 10% (PHIE > 0.1), shale volume less than 50% (VSHALE < 0.5), and water saturation less than 50% (SW < 0.5).

# Core versus Log Porosity Crossplot:

# Summary Plot: